From EPLAN Schematic to TwinCAT Project

Complete Beckhoff/TwinCAT control implementation of the Jenoptik "Funktionsmuster LCLW" machine

Sources: EPLAN electrical schematic (90 pages) + Festo pneumatic circuit diagram

Platform: TwinCAT XAE Shell (64-bit), simulation only — no physical hardware

Student: Ahmed Zakaria Benadda                                           Date: 27/10/2026

Lemsi Adams Favour

# 1. Introduction

This project rebuilds the complete control system of a Jenoptik machine prototype ("LCLW") as a TwinCAT 3 project, based on two original source documents: a 90-page EPLAN electrical schematic and a Festo pneumatic circuit diagram covering the same machine. Together they describe a conveyor-based part-handling machine that detects, clamps, inspects, and sorts parts along an infeed-to-outfeed line, with a central safety interlock and two reject paths.

All nine mechanical stations documented across both drawings have been implemented, chained into a single working process sequence, and verified by forcing I/O values live in TwinCAT with no physical hardware connected.

# 2. Hardware Architecture

Every control device identified across both drawings is a Beckhoff EtherCAT component:

| Device | Function | Eplan Designation |
| --- | --- | --- |
| C6015 | Embedded IPC — EtherCAT master, runs TwinCAT | +MP-4A1 |
| CU1128 | 8-port EtherCAT switch | +MP-5K1 |
| EK1100 (x2) | EtherCAT coupler — cabinet / CW-laser controller | +MP-6K1 / +LS-7K1 |
| EL1809 (x2) | 16-channel digital input terminal | +MP-6K10 / +LS-7K10 |
| EL2809 (x2) | 16-channel digital output terminal | +MP-6K20 / +LS-7K20 |
| EP1018-0001 (x3) | IP67 remote input box, 8DI — field-mounted | +MS-9K1 / 9K2 / 9K3 |
| EP2028-0001 (x2) | IP67 remote output box, 8DO — field-mounted | +MS-10K1 / 10K2 |
| AX5206 | 2-axis servo drive, 400 V | +MP-11T1 |
| AM8033 (x2) | Servo motors | +MS-11M1 / 11M2 |

The pneumatic drawing adds Festo valve-terminal and cylinder part numbers (VUVG series valves, ADN-series cylinders, SMC CUJ4-6 for the ejector) but introduces no new electrical signals — every cylinder it shows is driven by the digital outputs already present in the EPLAN schematic.

# 3. Machine Process Overview

The nine stations, in process order, with the two outcomes after the sorting decision:

| Einlauf | → | Zylinder Einlauf | → | Spanner (clamp) | → | Weiche (decision) | → |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | ↗ NIO | Auswerfer | → | Ausblasen (1s pulse) |
| --- | --- | --- | --- | --- |
|  | ↘ OK | Zylinder Auslauf | → | Auslauf (confirmed) |

Auswurf Bypass is a tenth, standalone station — no sensor or signal in either drawing defines what triggers it, so it is implemented independently with its own placeholder trigger (see Section 6.3).

# 4. TwinCAT Project Structure

## 4.1 EtherCAT Device Tree

I/O └─ Devices └─ EtherCAT_Master ├─ EK1100_6K1 │ ├─ Term 2 (EL1809) — cabinet digital inputs │ └─ Term 3 (EL2809) — cabinet digital outputs ├─ EP1018_9K1 / 9K2 / 9K3 — field input boxes ├─ EP2028_10K1 / 10K2 — field output boxes ├─ AX5206_11T1 — servo drive (2 axes, NC config) └─ EK1100_7K1 ├─ EL1809_7K10 — CW/laser controller inputs └─ EL2809_7K20 — CW/laser controller outputs

All devices set Disabled = Yes — no physical hardware connected (pure simulation).

## 4.2 Complete I/O Variable List

### Safety

| Variable | Tag | Description |
| --- | --- | --- |
| xNotHalt_Betaetigt | E0.1 | Emergency stop active |
| xLaserEnable | A0.1 | LaserEnable output |

### Einlauf (infeed)

| Variable | Tag | Description |
| --- | --- | --- |
| xLichtschranke_Einlauf | E1052.0 | Infeed photoelectric sensor |
| xServo_Einlauf_Eingerastet | E1052.1 | Infeed servo engaged (sensor) |
| xServo_Einlauf_AS | E1052.2 | Infeed servo working position |
| xServo_Einlauf_GS | E1052.3 | Infeed servo home position |
| xEinlaufBereit | — | Calculated: part ready (latch) |

### Zylinder Einlauf

| Variable | Tag | Description |
| --- | --- | --- |
| xZylinderEinlaufAS / GS | E1051.0 / .1 | Infeed cylinder position feedback |
| xVentilEinlaufAS / GS | A1051.0 / .1 | Infeed cylinder valve control |
| xZylinderEinlaufBereit | — | Calculated: cylinder confirmed in position |

### Spanner (clamp)

| Variable | Tag | Description |
| --- | --- | --- |
| xZylinderSpannerAS / GS | E1050.2 / .3 | Clamp cylinder position feedback |
| xVentilSpannerAS / GS | A1050.2 / .3 | Clamp valve control |
| xSpannerBereit | — | Calculated: clamp confirmed closed |

### Weiche (diverter) and sorting decision

| Variable | Tag | Description |
| --- | --- | --- |
| xPartOK | — | PLACEHOLDER: external quality decision (not present in either drawing) |
| xZylinderWeicheAS / GS | E1050.4 / .5 | Diverter cylinder position feedback |
| xVentilWeicheAS / GS | A1050.4 / .5 | Diverter valve control |
| xWeicheRejectBereit | — | Calculated: diverter confirmed reject position |
| xWeicheAcceptBereit | — | Calculated: diverter confirmed accept position |

### Auswerfer (ejector) + Ausblasen (blow-off) — reject path

| Variable | Tag | Description |
| --- | --- | --- |
| xZylinderAuswerferAS / GS | E1050.0 / .1 | Ejector cylinder position feedback |
| xVentilAuswerferAS / GS | A1050.0 / .1 | Ejector valve control |
| xAuswerferBereit | — | Calculated: ejector confirmed extended |
| xVentilAusblasenAS / GS | A1051.4 / .5 | Blow-off valve control (no feedback sensor) |

### Zylinder Auslauf + Auslauf — accept path

| Variable | Tag | Description |
| --- | --- | --- |
| xZylinderAuslaufAS / GS | E1051.2 / .3 | Outfeed cylinder position feedback |
| xVentilAuslaufAS / GS | A1051.2 / .3 | Outfeed cylinder valve control |
| xZylinderAuslaufBereit | — | Calculated: outfeed cylinder confirmed |
| xLichtschranke_Auslauf | E1052.4 | Outfeed photoelectric sensor |
| xServo_Auslauf_Eingerastet | E1052.5 | Outfeed servo engaged (sensor) |
| xServo_Auslauf_AS | E1052.6 | Outfeed servo working position |
| xServo_Auslauf_GS | E1052.7 | Outfeed servo home position |
| xAuslaufBereit | — | Calculated: part confirmed exited |

### Auswurf Bypass — standalone

| Variable | Tag | Description |
| --- | --- | --- |
| xBypassTrigger | — | PLACEHOLDER: no defining condition in either drawing |
| xVentilAuswurfBypassAS / GS | A1051.6 / .7 | Bypass valve control |

# 5. PLC Software Architecture

One Function Block per mechanical station, mirroring the one-page-per-station structure of the EPLAN drawing itself. Each FB reads/writes the shared GVL_IO variable list and is called once per cycle from MAIN, in physical process order.

## 5.1 MAIN

PROGRAM MAIN VAR fbEinlauf : FB_Einlauf; fbZylinderEinlauf : FB_ZylinderEinlauf; fbSpanner : FB_Spanner; fbWeiche : FB_Weiche; fbAuswerfer : FB_Auswerfer; fbAusblasen : FB_Ausblasen; fbZylinderAuslauf : FB_ZylinderAuslauf; fbAuslauf : FB_Auslauf; fbAuswurfBypass : FB_AuswurfBypass; END_VAR GVL_IO.xLaserEnable := NOT GVL_IO.xNotHalt_Betaetigt; fbEinlauf(); fbZylinderEinlauf(); fbSpanner(); fbWeiche(); fbAuswerfer(); fbAusblasen(); fbZylinderAuslauf(); fbAuslauf(); fbAuswurfBypass();

## 5.2 FB_Einlauf — infeed detection (SET/RESET latch)

IF GVL_IO.xLichtschranke_Einlauf AND GVL_IO.xServo_Einlauf_GS THEN GVL_IO.xEinlaufBereit := TRUE; ELSIF NOT GVL_IO.xLichtschranke_Einlauf THEN GVL_IO.xEinlaufBereit := FALSE; END_IF

## 5.3 FB_ZylinderEinlauf

GVL_IO.xVentilEinlaufAS := GVL_IO.xEinlaufBereit; GVL_IO.xVentilEinlaufGS := NOT GVL_IO.xEinlaufBereit; GVL_IO.xZylinderEinlaufBereit := GVL_IO.xZylinderEinlaufAS;

## 5.4 FB_Spanner

GVL_IO.xVentilSpannerAS := GVL_IO.xZylinderEinlaufBereit; GVL_IO.xVentilSpannerGS := NOT GVL_IO.xZylinderEinlaufBereit; GVL_IO.xSpannerBereit := GVL_IO.xZylinderSpannerAS;

## 5.5 FB_Weiche — sorting decision

GVL_IO.xVentilWeicheAS := GVL_IO.xSpannerBereit AND NOT GVL_IO.xPartOK; GVL_IO.xVentilWeicheGS := GVL_IO.xSpannerBereit AND GVL_IO.xPartOK; GVL_IO.xWeicheRejectBereit := GVL_IO.xZylinderWeicheAS; GVL_IO.xWeicheAcceptBereit := GVL_IO.xZylinderWeicheGS;

## 5.6 FB_Auswerfer

GVL_IO.xVentilAuswerferAS := GVL_IO.xWeicheRejectBereit; GVL_IO.xVentilAuswerferGS := NOT GVL_IO.xWeicheRejectBereit; GVL_IO.xAuswerferBereit := GVL_IO.xZylinderAuswerferAS;

## 5.7 FB_Ausblasen — first timer (TON)

// Declaration: tonBlow : TON; // Implementation: tonBlow(IN := GVL_IO.xAuswerferBereit, PT := T#1S); GVL_IO.xVentilAusblasenAS := GVL_IO.xAuswerferBereit AND NOT tonBlow.Q; GVL_IO.xVentilAusblasenGS := NOT GVL_IO.xVentilAusblasenAS;

TON (Timer On-delay): IN starts the countdown; once PT (here 1 second) elapses while IN stays TRUE, output Q becomes TRUE. Here, Q flipping TRUE is what cuts the blow valve back off automatically, one second after the ejector confirms — no further input needed.

## 5.8 FB_ZylinderAuslauf

GVL_IO.xVentilAuslaufAS := GVL_IO.xWeicheAcceptBereit; GVL_IO.xVentilAuslaufGS := NOT GVL_IO.xWeicheAcceptBereit; GVL_IO.xZylinderAuslaufBereit := GVL_IO.xZylinderAuslaufAS;

## 5.9 FB_Auslauf

IF GVL_IO.xLichtschranke_Auslauf AND GVL_IO.xServo_Auslauf_GS THEN GVL_IO.xAuslaufBereit := TRUE; ELSIF NOT GVL_IO.xLichtschranke_Auslauf THEN GVL_IO.xAuslaufBereit := FALSE; END_IF

## 5.10 FB_AuswurfBypass

// Declaration: tonBypass : TON; // Implementation: tonBypass(IN := GVL_IO.xBypassTrigger, PT := T#1S); GVL_IO.xVentilAuswurfBypassAS := GVL_IO.xBypassTrigger AND NOT tonBypass.Q; GVL_IO.xVentilAuswurfBypassGS := NOT GVL_IO.xVentilAuswurfBypassAS;

# 6. Design Notes

## 6.1 SET/RESET latch pattern

Used in FB_Einlauf and FB_Auslauf: a condition sets a flag TRUE and it holds even as other inputs change, mirroring a part physically staying detected as it moves further into the machine — only clearing once the relevant light curtain goes clear again.

## 6.2 Feedback-gated chaining

Every actuator station both drives its valve from the upstream condition AND separately reports its own confirmed position back as a *Bereit ("ready") flag, calculated from its own position sensor — never from the command it was just given. This means each next station only fires once the previous one has physically confirmed arrival, not merely been commanded — the same principle a real safety-rated sequence would require.

## 6.3 Two placeholder variables

xPartOK and xBypassTrigger represent decisions neither source drawing defines (a quality/vision check, and whatever condition sends a part to bypass reject). Both are implemented as plain forced variables, clearly commented, standing in for signals that would come from equipment outside the scope of these two drawings.

# 7. Safety Interlock

GVL_IO.xLaserEnable := NOT GVL_IO.xNotHalt_Betaetigt;

In the real machine this runs through a certified hardware safety relay, not PLC logic alone — this line models the intent for simulation purposes only.

# 8. Testing and Verification

All testing was performed with no physical hardware connected, by forcing GVL_IO variables live through TwinCAT's online Watch/Force interface and observing dependent outputs update once per PLC cycle.

| Chain tested | Result |
| --- | --- |
| Not-Halt pressed / released → LaserEnable | Pass |
| Infeed sensors → Einlauf latch → cylinder command | Pass |
| Latch holds after GS clears, resets after Lichtschranke clears | Pass |
| Cylinder confirms → clamp commanded → clamp confirms | Pass |
| xPartOK = TRUE → Weiche accept path commanded | Pass |
| xPartOK = FALSE → Weiche reject path commanded | Pass |
| Diverter confirms reject → Auswerfer commanded → confirms → Ausblasen fires | Pass |
| Ausblasen auto-cuts off after 1s via TON | Pass |
| Diverter confirms accept → Zylinder Auslauf commanded → confirms | Pass |
| Outfeed sensors confirm part fully exited (xAuslaufBereit) | Pass |
| Auswurf Bypass fires on trigger, auto-cuts off after 1s | Pass |

GVL_IO online view during the full-chain test sequence :

EtherCAT device tree and full POUs list in Solution Explorer.

# 9. Conclusion

This implementation covers the complete process documented across both source drawings: nine mechanical stations plus the safety interlock, structured as independent Function Blocks chained through feedback-confirmed state, including a genuine sorting decision and two timer-based pulse outputs. The pattern is fully consistent across every station, directly traceable back to specific tags on the original EPLAN pages, and was verified end-to-end by simulated forcing from infeed sensor through to outfeed confirmation on both the accept and reject paths.